| Test | Specification |  | Reporting Value |
| --- | --- | --- | --- |
| Meets NF Requirements |  |  |  |
| Description |  | Transparent, colorless, mobile, Volatile liquid having a characteristic odor. | Complies |
| Solubility |  | Miscible with water, with alcohol, with ether, with chloroform and with most volatile oils. | Complies |
| Identification A (By IR) |  | The IR absorption spectrum of the standard should match with that of IR absorption spectrum of sample. | Complies |
| Identification B (By GC) |  | The retention time of the major peak in the chromatograph of the sample corresponds to that in the chromatograph of USP Acetone RS as obtained in the test for Assay | Complies |
| Specific Gravity at 25 ° C |  | Not more than 0.789 | 0.788 |
| Nonvolatile Residue |  | The weight of the residue does not exceed 2mg (0.004 %) | 0.0004% |
| Readily Oxidizable Substances |  | The permanganate color of the mixture does not completely disappear within 15mins | Complies |
| Water (H2O) |  | Not more than 0.5 % | 0.15% |

| Test | Specification | Reporting Value |
| --- | --- | --- |
| Assay (as (CH3)2CO)(calc on anhydrous basis) | NLT 99.0 % | 100.1% |
| Benzene | Not more than.2 ppm | Not Detected |
| Meets EP/BP Chemical Specifications |  |  |
| Appearance | Volatile, clear, colourless liquid | Complies |
| Solubility | Miscible with water and ethanol (96 percent) | Complies |
| Identification A (By Relative density) | 0.790-0.793 | 0.791 |
| Identification B (By Chemical test) | An intense red colour is produced which becomes violet with the addition of 3.5 mL of acetic acid R. | Complies |
| Identification C (By chemical test) | A greenish-blue colour is produced | Complies |
| Appearance of Solution | The solution is cleat and colorless | Complies |
| Acidity or Alkalinity | The solution is red or orange | Complies |
| Density (g/mL)at 20 ° C | 0.790-0.793 | 0.791 |
| Related Substances |  |  |
| Impurity A | Not more than 0.05 % | 0.012% |
| Impurity B | Not more than 0.05 % | 0.0015 |
| Impurity C | Not more than 2 ppm | Not Detected |
| Any other impurity | Not more than 0.05 % | 0.004% |
| Matter Insoluble in Water | The solution is clear | Passes Test |

| Test | Specification | Reporting Value |
| --- | --- | --- |
| Reducing Substances | The mixture is not completely decolourised. | Passes Test |
| Residue on Evaporation | Maximum 50 ppm | 25ppm |
| Water (H2O), g/L | Maximum 3 g/L | 2.2 g/l |