| Test | Specification | Results |
| --- | --- | --- |
| Meets NF Requirements |  |  |
| Description | A Clear colorless, mobile liquid having odor resembling that of chloroform | Complies |
| Solubility | Miscible with alcohol, ether and fixed oils | Complies |
| Assay | Not less than 99.0% | 100.0% |
| Identification by IR | The IR absorption spectrum of the vapor shows strong doublet peaks at 7.8 and 7.9 µm and at 13.2 and 13.4 µm, and relatively few minor peaks | Complies |
| Free chlorine (Cl2) | The lower layer does not show a violet tint | Complies |
| Hydrogen Chloride (HCl) | NMT 0.001%; the pink color in the sample cylinder is at least as intense as that in the comparison cylinder, and the color persists for NLT 15 min | <0.001% |
| Non Volatile Residue | NMT 0.002%; NMT 1 mg of residue | 0.002% |
| Specific Gravity at 25c | 1.318 - 1.322 | 1.319 |
| Water | Not more than 0.02 % | 0.01% |
| Meets EP/BP Chemical Specifications |  |  |
| Description | A Clear colorless, volatile liquid. | Complies |
| Solubility | Sparingle soluble in water, miscible with alcohol | Complies |
| Identification B (Refractive Index) | 1.423-1.425 | 1.423 |
| Identification C (By IR) | The sample spectrum should match that of the reference standard | Complies |
| Appearance | It is clear and colourless. | Complies |

| Test | Specification | Results |
| --- | --- | --- |
| Acidity | Not more than 0.15 mL of 0.1 M sodium hydroxide is required to change the colour of the indicator to blue. | Complies |
| Relative Density | 1.320 - 1.332 | 1.321 |
| Refractive Index | 1.423 - 1.425 | 1.423 |
| Ethanol, 2-methylbut-2-ene, and volatile impurities: |  |  |
| Ethanol | Maximum 2.0 % | Not Detected |
| 2-Methylbut-2-ene | Maximum 300 ppm | 14 ppm |
| Impurity A | Maximum 10 ppm | Not Detected |
| Impurity B | Maximum 50 ppm | 7 ppm |
| Total of Impurities other than ethanol and 2- methybut-2-ene | Maximum 0.1 % | Not Detected |
| Free Chlorine (Cl2) | No blue colour develops | Complies |
| Residue on evaporation | Maximum 20 ppm | 4 ppm |
| Water | Maximum 0.02 % | 0.01% |