| Test | Specification | Results |
| --- | --- | --- |
| Meets EP/BP Chemical Specifications |  |  |
| Description | A Clear colorless, volatile liquid. | Complies |
| Solubility | Sparingle soluble in water, miscible with alcohol | Complies |
| Identification B (Refractive Index) | 1.423-1.425 | 1.423 |
| Identification C (By IR) | The sample spectrum should match that of the reference standard | Complies |
| Appearance | It is clear and colourless. | Complies |
| Acidity | Not more than 0.15 mL of 0.1 M sodium hydroxide is required to change the colour of the indicator to blue. | Complies |
| Relative Density | 1.320 - 1.332 | 1.322 |
| Refractive Index | 1.423 - 1.425 | 1.423 |
| Ethanol, 2-methylbut-2-ene, and volatile impurities: |  |  |
| Ethanol | Maximum 2.0 % | Not Detected |
| 2-Methylbut-2-ene | Maximum 300 ppm | 24.12 ppm |
| Impurity A | Maximum 10 ppm | Not Detected |
| Impurity B | Maximum 50 ppm | Not Detected |

| Test | Specification | Results |
| --- | --- | --- |
| Total of Impurities other than ethanol and 2- methybut-2-ene | Maximum 0.1 % | Not Detected |
| Free Chlorine (Cl2) | No blue colour develops | Passes Test |
| Residue on evaporation | Maximum 20 ppm | 10ppm |
| Water | Maximum 0.02 % | 0.01% |