| Test | Specification | Reporting Value |
| --- | --- | --- |
| Meets NF Requirements |  |  |
| Description | Transparent, colorless, mobile, Volatile liquid having a characteristic odor. | Complies |
| Solubility | Miscible with water, with alcohol, with ether, with chloroform and with most volatile oils. | Complies |
| Identification A (By IR) | The IR absorption spectrum of the standard should match with that of IR absorption spectrum of sample. | Complies |
| Identification B (By GC) | The retention time of the major peak in the chromatograph of the sample corresponds to that in the chromatograph of USP Acetone RS as obtained in the test for Assay | Complies |
| Specific Gravity at 25 ° C | Not more than 0.789 | 0.782 |
| Nonvolatile Residue | The weight of the residue does not exceed 2mg (0.004 %) | 0.0022% |
| Readily Oxidizable Substances | The permanganate color of the mixture does not completely disappear within 15mins | Complies |
| Water (H2O) | Not more than 0.5 % | 0.11% |
| Assay (as (CH3)2CO)(calc on anhydrous basis) | NLT 99.0 % | 100.1% |
| Benzene | Not more than.2 ppm | Not Detected |